EXP 03  5G NR Throughput for QPSK–256QAM

AIM :

To calculate and compare 5G NR throughput for different modulation schemes (QPSK, 16QAM, 64QAM, 256QAM) using parameters such as bandwidth, subcarrier spacing, coding rate, and MIMO layers.

THEORY :

5G NR throughput depends on modulation order, coding rate, number of subcarriers, and MIMO layers.
Higher-order modulations carry more bits per symbol and provide higher data rates.
Throughput is estimated using:

T=(bitsPerSymbol)×Nsub×codeRate×MIMO_layers

Where:

- Bits per symbol: QPSK=2, 16QAM=4, 64QAM=6, 256QAM=8

- Nsub=BWSCS

- Higher modulation → higher throughput.

PROCEDURE :

- Set 5G NR parameters: BW=100 MHz, SCS=30 kHz, code rate=0.85, MIMO layers=4.

- Compute total subcarriers using BW/SCS.

- Define modulation schemes and bits per symbol.

- Calculate throughput using the formula for each modulation.

- Display results and plot throughput vs. modulation.

PROGRAM :

clear; clc; close all;

%% 5G NR Parameters

BW = 100e6;             % Bandwidth (100 MHz)

subcarrierSpacing = 30e3; % 30 kHz SCS

Nsub = BW / subcarrierSpacing; % Total subcarriers

codeRate = 0.85;        % Example coding rate

MIMO_layers = 4;        % 4x4 MIMO

%% Modulation Schemes & Bits per Symbol

mod_scheme = {'QPSK','16QAM','64QAM','256QAM'};

bits_per_symbol = [2 4 6 8];   % bps for each modulation

%% Throughput Calculation

throughput = zeros(1,length(bits_per_symbol));

for i = 1:length(bits_per_symbol)

bps = bits_per_symbol(i);

% Throughput formula:

% T = (bits per symbol) * (no. of subcarriers) * (coding rate) * (MIMO layers)

T = bps * Nsub * codeRate * MIMO_layers;

throughput(i) = T / 1e6;   % Convert to Mbps

end

%% Display Results

disp('5G NR Throughput for Different Modulation Schemes:');

for i=1:length(mod_scheme)

fprintf('%s Throughput = %.2f Mbps\n', mod_scheme{i}, throughput(i));

end

%% Plot Throughput

figure;

bar(throughput,0.6);

set(gca,'XTickLabel',mod_scheme,'FontSize',12,'FontWeight','bold');

xlabel('Modulation Scheme'); ylabel('Throughput (Mbps)');

title('5G NR Throughput for QPSK–256QAM');

grid on;

OUTPUT :

CONSOLE OUTPUT :

5G NR Throughput for Different Modulation Schemes:
QPSK Throughput = 0.02 Mbps
16QAM Throughput = 0.05 Mbps
64QAM Throughput = 0.07 Mbps
256QAM Throughput = 0.09 Mbp

CONCLUSION :

Throughput increases as modulation order increases. QPSK gives the lowest throughput, while 256QAM gives the highest. This shows that high-order modulation and MIMO significantly improve 5G NR data rates.